# Smart Water Monitoring System

Author: Talha

# Introduction

Purpose & Problem :
Water pipeline leaks result in significant resource loss—globally, utilities lose an estimated 15–30% of treated water before it reaches consumers. These leaks often go undetected until they cause costly damage and service disruption. Our project delivers a real-time, geolocated leak detection system, combining IoT sensing, machine learning, and GIS mapping to empower municipalities and facility managers with faster, data-driven response capabilities.

Solution Overview :

- Sensors measure key parameters (pressure, flow, temperature, vibration, motor RPM) and transmit data (via ESP8266 or similar) to a backend server.

- A Flask API ingests this sensor data in real-time and applies a trained XGBoost ML model to detect leaks.

- A Streamlit dashboard visualizes predictions and logs events on an interactive GIS map.

Key Results

- The XGBoost model shows strong performance (e.g., accuracy ≥  99 %, ROC-AUC ≥  100%) on labeled test data.

- Rapid location-based leak detection potentially reduces non-revenue water loss and speeds maintenance responses.

- All components are designed for local operation, with CSV-based logging—no cloud dependencies.

Recommendations & Next Steps :

- Integrate SMS/email alerting for immediate response notification.

- Connect real sensor hardware (ESP8266 + sensors), calibrate thresholds in-field.

- Upgrade storage to cloud platforms (e.g., Firebase, SQL) for broader scalability.

- Containerize components with Docker and implement CI/CD pipelines.

- Add model monitoring to detect drift and ensure long-term model robustness.

Commercial & Social Impact :

This system enables early leak detection, reducing water waste and enabling targeted repair interventions. Adopting this solution supports sustainable water management, operational efficiency, and improved infrastructure reliability—especially critical in aging urban water network.

2. Dataset & Preprocessing :

-  Dataset : `location_aware_gis_leakage_dataset.csv`

-  Features :

- `pressure`

- `flow rate`

- `temperature`

- `vibration`

- `motor rpm`

- `operational hours`

- `latitude`, `longitude`

- `leakage detected` (target variable)

Preprocessing Steps :

- Handled missing values

- Feature scaling using `scaler.joblib`

- Train–test split and label encoding

Sample Data Snapshot :

3. Feature Engineering & Model Training

Input Features :

Core sensor readings + coordinates

Model Used :

XGBoost classifier, trained via `model_training.py`

Artifacts Output :

`xgboost_model.joblib`

`scaler.joblib`

4. Model Evaluation :

Accuracy : 99%

Precision / Recall / F1-Score : 98%/96%/97%

ROC-AUC : 100%

ROC curve and confusion matrix images:

5. Deployment & API Design :

Framework : Flask (`app.py`)

Endpoint : `/predict`

Input : JSON with sensor data + latitude/longitude

Output : leak probability, leak label, timestamp

Logging : Results stored in `leakage_log.csv`

6. Dashboard & Visualization

Front-end : Streamlit (`dashboard.py`)

Features :

- Real-time sensor input

- Leak prediction output

- GIS map of leak event logs

- Data Source: Local CSV log file

7. Hardware & System Architecture

Sensors Used : Flow, pressure, vibration, motor RPM, temperature

Controller :  ESP8266 (assumed) sending data to Flask API

System Flow :

[IoT Sensors] → [ESP8266] → [Flask API] → [XGBoost Model] → [Streamlit UI & CSV Logs]

---

8. Cloud Integration & Alerting

Current Storage : Local CSV files

Potential Enhancements :

Add email/SMS alerting

Migrate to cloud storage solutions (e.g. Firebase, SQL database)

---

9. Deployment Guide

1. Install dependencies:

pip install -r requirements.txt

Train the model:

python model_training.py

Launch API:

python app.py

Run the dashboard:

streamlit run dashboard.py

Intended Users: IoT/data engineers, utility operators, facility managers

Calibration Required: Real-world sensor thresholds should be tuned based on environment

10. Business Value & Impact :

- Enables early leak detection, reducing water loss

- Provides geospatial insights for targeted maintenance

- Cuts operational costs and enhances sustainability

- Supports proactive, data-driven utility management

11. Limitations & Future Work :

- No real hardware integration yet

- Alerting system not yet implemented

- Storage is local—no cloud scalability

- Model monitoring and drift detection yet to be added

- Recommend containerization (Docker) and CI/CD pipelines for production readiness

Appendix :

- Feature Dictionary :

| Feature | Description | Unit |
| --- | --- | --- |
| pressure | Water pressure | psi |
| flow_rate | Flow rate | m³/s |
| temperature | Sensor temperature | °C |
| vibration | Vibration reading | m/s² |
| motor_rpm | Motor RPM | RPM |
| operational_hours | Operational duration | Hours |
| latitude, longitude | Geo-coordinates of sensor | Decimal ° |
| leakage_detected | Leak status label (0/1) | Binary |

- Key Code Snippets:

- Model code Snippets:

- App code Snippets:

- Dashboard Code Snippets:

- References

Project best practices and documentation frameworks